Lab Report

Date: June 1st , 2026

Team: E1

Team Members

| Last name | First name | Email |
| --- | --- | --- |
| Rahman | Md Mahbubur | md-mahbubur.rahman@stud.hshl.de |
| Sakib | Md Naimur Rahaman | md-naimur-rahaman.sakib@stud.hshl.de |
| Xu | Yongyu | yongyu.xu@stud.hshl.de |
| Koluacik | Baris | baris.koluacik@stud.hshl.de |

Goal

- 1.Connect the wires between Arduino, breadboard, motor controller, and battery, so the vehicle can be  able to follow the lane.

- Discussion

- 1. We uploaded the code that worked fine in Tinkercad to the Arduino, wired everything up, connected the battery, and flipped the switch, but the vehicle didn’t respond at all.

- After repeated checks, we found:

- 1.1 The breadboard was wired incorrectly, and some parts weren’t receiving power.

- 1.2 The positive and negative terminals of the motor were connected incorrectly.

- 2. After multiple adjustments, the motor finally spun normally, but we couldn’t control its speed. So, we checked again and found:

- 2.1 We realized that not all digital pins can output PWM. So we looked for the tilde symbol (~) next to the pin number on Arduino (e.g., pins 3, 5, 6, 9, 10, and 11), and adjusted the wires.

- Finally, the vehicle was able to move slowly. For details, please refer to the video and codes in the folder.

- 3. Currently, our vehicle cannot detect black lines or obstacles. We believe this may be because the ultrasonic sensors and IR sensors are interfering with each other.

- We will adjust the code to use the IR sensors to detect the lane first, and then use the ultrasonic sensors to detect obstacles.